Project Report

On

# Design & Veriﬁcation of Single Port RAM

# By

# Swayam Swastik Sahu

Department of Electronics and Communication Engineering

NIST University

Institute Park, Berhampur, Odisha -761008, India

www.nist.edu

## Table of Contents

- Abstract

- Introduction

- Design Overview

- Verilog Code

- Testbench Design

- Simulation and Results

- Conclusion

- References

- Appendix

## Abstract

This project involves the design and verification of a 64x8-bit single-port Random Access Memory (RAM) using Verilog. The RAM module is synchronous, with both read and write operations controlled by a clock signal and a write enable (we) signal. Various test cases were implemented in a Verilog testbench to verify the correct functionality of the RAM, including basic read/write operations, overwriting data, random access, and checking for uninitialized memory. The results of the simulation demonstrate that the single-port RAM design performs as expected, meeting the project’s objectives.

## Introduction

#### Background

In digital systems, Random Access Memory (RAM) is essential for temporary data storage, allowing fast access and retrieval of data. A single-port RAM, in particular, allows both read and write operations to be performed, but only one operation at a time through a single address bus. This type of RAM is commonly used in embedded systems, processors, and data buffering applications where memory access can be controlled sequentially.

As part of this project, a single-port RAM was designed using Verilog, a hardware description language (HDL) used to model digital circuits. The main goal of this project was to implement a 64x8-bit single-port RAM module, ensuring both read and write operations are correctly performed.

    - RAM is considered as volatile memory, which means that the stored information is lost when there is no power.

    - RAM is made up of small memory chips that form a memory module.

    - Whenever you open a program, it gets loaded from the hard drive into the RAM.

    - This is because reading data from the RAM is much faster than reading data from the hard drive.

#### Objective

The objective of this project is to:

- Design	a	single-port	RAM	with	64	memory locations, each capable of storing 8-bit data.

- Simulate the design to verify the correct functionality using various test cases.

#### Scope

This project focuses on the design and verification of a basic single-port RAM using Verilog. The memory can store 64 locations of 8-bit data, and a clock-driven mechanism is used for synchronous read/write operations. The scope of this project does not include advanced features like dual-port RAM, multi-bank memory, or error-correcting code (ECC) implementation.

## Design Overview

  - Single-Port RAM Architecture

The single-port RAM allows either a read or a write operation at a given time. The architecture includes:

- Data Bus (8-bit): For reading from and writing to memory.

- Address	Bus	(6-bit):	To	specify	the	memory	location	for read/write operations.

- Write Enable (we): A control signal to determine whether to read from or write to memory.

- Clock Signal: Used to synchronize all operations.

Block diagram:

  - Key Features:

- Data Width: 8-bit data for each memory location.

- Address Width: 6-bit address, enabling access to 64 locations.

- Write Enable: A control signal that allows data to be written when high (we = 1), or read when low (we = 0).

- Synchronous Operation: Both read and write operations are synchronous with the clock signal.

## Verilog Code

Single-Port RAM Design Code:

Code Explanation:

- Module Definition (single_port_ram): This is a Verilog module that implements a single-port RAM.

- Inputs:

  - data [7:0]: 8-bit input data to be written into the RAM.

  - addr	[5:0]: 6-bit address input, allowing access to 64 memory locations.

  - we: Write enable signal. When high, data is written to the RAM at the specified address.

  - clk: Clock signal, used to trigger the data writing and address storing operations.

- Output:

  - q [7:0]: 8-bit output, which holds the data read from the RAM.

- RAM Declaration:

  - reg [7:0] ram [63:0]: A 64 x 8-bit memory array is declared, where each of the 64 memory locations stores 8-bit data.

- Address Register:

  - reg [5:0] addr_reg: A register to store the address during a read operation. It holds the address from which data is read in the next cycle.

- Sequential Block:

  - The always @(posedge clk) block triggers on the rising edge of the clock.

  - Write Operation:

    - If the we (write enable) signal is high (1), the input data is written into the memory location specified by the addr.

  - Read Operation:

    - If we is low (0), the current address value from addr is stored in addr_reg.

- Data Output Assignment:

  - The output q is continuously assigned the data stored in the memory location addressed by addr_reg from the last read cycle.

##### RTL Schematic:

Figure 1: Schematic of Single Port RAM

- Testbench Design

Testbench Code:

Code Explanation:

- Module Declaration:

  - The  module  single_port_ram_tb  is  declared  for  testing the single_port_ram module.

- Inputs/Outputs:

  - reg   [7:0]   data: 8-bit register to hold the input data for the RAM.

  - reg	[5:0]	addr:  6-bit  register  to  hold  the  address  to

access the RAM locations.

  - reg we: A register for the write enable signal.

  - reg  clk: A register for the clock signal.

  - wire   [7:0]   q: Wire to capture the 8-bit output from the RAM.

- Instantiation of the RAM:

  - The   single_port_ram   module   is   instantiated   as   SPR, connecting the input and output signals to the testbench’s data, addr, we, clk, and q.

Clock Generation:

- Clock Initialization:

  - clk=1'b1;: The clock is initialized to 1 (high) at the start.

  - forever   #5   clk   =   ~clk;: The clock toggles every 5 time units, creating a clock period of 10 time units.

Test Cases:

  - The initial  block defines various test scenarios to verify the correct functionality of the single-port RAM.

Test Case 1: Basic Write and Read Operation

- Write 8'hAA to address 0.

- Write 8'hBB to address 1.

- Read from address 0 and 1 by setting we	= 0 (disabling write mode).

###### Test Case 2: Multiple Write Operations Followed by Read

- Write	8'h11,	8'h22,	and  8'h33   to  addresses  0,  1,  and  2, respectively.

- Read the data back from addresses 0, 1, and 2.

###### Test Case 3: Overwrite Data at a Specific Address

- Write 8'h44 to address 3.

- Overwrite with 8'h55 at the same address 3.

- Read the value from address 3 to verify the overwrite.

###### Test Case 4: Write and Read at Random Addresses

- Write 8'h66 to address 5 and 8'h77 to address 10.

- Read from the same addresses (5 and 10) to verify the data.

###### Test Case 5: Check RAM for Uninitialized Values

- Check data from address 7 and address 15 without initializing them (to test uninitialized memory locations).

###### Test Case 6: Simultaneous Read/Write to the Same Address

- Write  8'hAA  to  address  8  and  then  immediately  overwrite with

8'hBB at the same address.

- Read the data from address 8 to check if the overwrite was successful.

- Ending the Simulation:

  - After running the test cases, the simulation is stopped with

$stop after a delay of 20ns.

This testbench simulates different scenarios like write, read, overwriting, and random access operations to verify the functionality of a single-port RAM.

### Simulation Output

- Conclusion

The design and verification of a single-port RAM using Verilog was successfully completed. The RAM was tested under various conditions, and the design met the expected functionality, including correct read/write operations and random access. Future work could include expanding the design to dual-port RAM or adding error detection and correction mechanisms.

### References

- Tools : Vivado

- Hardware Description Language : Verilog

### Appendix

The full Verilog code for the Single-Port RAM implementation, including the main module and testbench, can be accessed on GitHub. For detailed code listings and further implementations, please visit the following repository:

GitHub Repository:

# THANK YOU !!